- Explain the Medium Access Control (MAC) problem of receiving various signals from exposed and hidden terminals and near and far terminals.

2) What is Multiplexing? Explain different multiplexing schemes?

A)  Multiplexing; Multiplexing is a way of sending multiple signals or streams of information over a communications link at the same time in the form of a single, complex signal.

- Process of different users sharing a same medium with minimum or no interference

- Needs guard space

- Guard space was given to prevent the interference between the various signals

Multiplexing in 4 dimensions

  - space (si)

  - time (t)

  - frequency (f)

  - code (c)

Space division multiplexing;

- Medium has been shared by various signals

- But each and every signal has differaent orientation

- Enough guard space need to be given

- Wastes lot of band width

- Example old analog telephone system.

- SDMA: Interference happens if senders are too close to each other. Terminals or base stations have to keep a minimum distance.

Frequency multiplexing;

- subdivide the frequency dimension into several non-overlapping frequency bands

- Senders using a certain frequency band can use this band continuously. Again, guard spaces are needed to avoid frequency band overlapping (also called adjacent channel interference).

- This scheme is used for radio stations within the same region, where each radio station has its own frequency

- Interference happens if senders transmit data at the same frequency.

Advantages:

- no dynamic coordination 
necessary

- works also for analog signals

Disadvantages:

- While radio stations broadcast 24 hours a day, mobile communication typically takes place for only a few minutes at a time.

- Wastes bandwidth

- Inflexible.

Time division multiplexing;

- Here a channel is given the whole bandwidth for a certain amount of time.

- all senders use the same frequency but at different points in time

- Again, guard spaces, which now represent time gaps, have to separate the different periods when the senders use the medium

- Interference happens if senders transmit data at the same time.

Code multiplex

3) Draw the system architecture of GSM; explain the various components associated with its subsystems.

GSM is a PLMN (Public Land Mobile Network)

  - several providers setup mobile networks following the GSM standard within each country

  - components

    - MS (mobile station)

    - BS (base station)

    - MSC (mobile switching center)

    - LR (location register)

  - subsystems

    - RSS (radio subsystem): covers all radio aspects

    - NSS (network and switching subsystem): call forwarding, handover, switching

    - OSS (operation subsystem): management of the network

Radio subsystem

The Radio Subsystem (RSS) comprises the cellular mobile network up to the switching centers

- Components

  - Base Station Subsystem (BSS):

    - Base Transceiver Station (BTS): radio components including sender, receiver, antenna - if directed antennas are used one BTS can cover several cells

    - Base Station Controller (BSC): switching between BTSs, controlling BTSs, managing of network resources, mapping of radio channels (Um) onto terrestrial channels (A interface)

    - BSS = BSC + sum(BTS) + interconnection

  - Mobile Stations (MS)

Mobile station:

Terminal for the use of GSM services

- A mobile station (MS) comprises several functional groups

  - MT (Mobile Terminal):

    - offers common functions used by all services the MS offers

    - corresponds to the network termination (NT) of an ISDN access

    - end-point of the radio interface (Um)

  - SIM (Subscriber Identity Module):

    - personalization of the mobile terminal, stores user parameters

- international mobile equipment identity(IMEI)

- a personal identity number (PIN),

- PIN unblocking key (PUK),

Base Transfer Station and Base Station Controller:

Tasks of a BSS are distributed over BSC and BTS

- BTS comprises radio specific functions

- BSC is the switching center for radio channels

Network and switching subsystem:

NSS is the main component of the public mobile network GSM

  - switching, mobility management, interconnection to other networks, system control

- Components

  - Mobile Services Switching Center (MSC)
controls all connections via a separated network to/from a mobile terminal within the domain of the MSC - several BSC can belong to a MSC

- MSC manages several BSCs in a geographical region. A gateway

MSC (GMSC) has additional connections to other fixed networks, such as PSTN and ISDN.

- Using additional interworking functions (IWF),

- An MSC handles all signaling needed for connection setup, connection release and handover of connections to other MSCs. The standard signaling system No. 7 (SS7) is used for this purpose. SS7

- Features of SS7 are number portability, free phone/toll/collect/credit calls, call forwarding, three-way calling etc

HLR;

- This comprises static information, such as the mobile subscriber ISDN number (MSISDN), subscribed services (e.g., call forwarding, roaming restrictions, GPRS), and the international mobile subscriber identity (IMSI).

- Dynamic information is also needed, e.g., the current location area (LA) of the MS, the mobile subscriber roaming number (MSRN), the current VLR and MSC

- As soon as an MS leaves its current LA, the information in the HLR is updated. This information is necessary to localize a user in the worldwide GSM network

VLR:

- The VLR associated to each MSC is a dynamic database which stores all important information needed for the MS users currently in the LA that is associated to the MSC (e.g., IMSI,MSISDN, HLR address).

- If a new MS comes into an LA the VLR is responsible for, it copies all relevant information for this user from the HLR

Operation subsystem:

- Operation and maintenance center (OMC): The OMC monitors and controls all other network entities via the O interface (SS7 with X.25).

- Typical OMC management functions are traffic monitoring, status reports of networkentities, subscriber and security management, or accounting and billing.

- OMCs use the concept of telecommunication management network(TMN) as standardized by the ITU-T.

Authentication centre (AuC)

- Authentication centre (AuC): As the radio interface and mobile stations are particularly vulnerable, a separate AuC has been defined to protect user identity and data transmission.

- The AuC contains the algorithms for authentication as well as the keys for encryption and generates the values needed for user authentication in the HLR.

- The AuC may, in fact, be situated in a special protected part of the HLR.

Equipment identity register

- Equipment identity register (EIR): The EIR is a database for all IMEIs, i.e., it stores all device identifications registered for this network

- . As MSs are mobile, they can be easily stolen. With a valid SIM, anyone could use the stolen MS. The EIR has a blacklist of stolen (or locked) devices.

- In theory an MS is useless as soon as the owner has reported a theft. Unfortunately, the blacklists of different providers are not usually synchronized and the illegal use of a device in another operator’s network is possible (the reader may speculate as to why this is the case).

The EIR also contains a list of valid IMEIs (white list), and a list of malfunctioning devices (gray list)

- With advantages and disadvantages, explain the characteristics of different orbits of satellites

Four different types of satellite orbits can be identified depending on the shape and diameter of the orbit:

- GEO: geostationary orbit,  ca. 36000 km above earth surface

- LEO (Low Earth Orbit): ca. 500 - 1500 km

- MEO (Medium Earth Orbit) or ICO (Intermediate Circular Orbit): ca. 6000 - 20000 km

- HEO (Highly Elliptical Orbit) elliptical orbits

Geostationary satellites

Orbit 35,786 km distance to earth surface, orbit in equatorial plane (inclination 0°)

complete rotation exactly one day, satellite is synchronous to earth rotation

- fix antenna positions, no adjusting necessary

- satellites typically have a large footprint (up to 34% of earth surface!), therefore difficult to reuse frequencies

- bad elevations in areas with latitude above 60° due to fixed position above the equator

- high transmit power needed

- high latency due to long distance (ca. 275 ms)

not useful for global coverage for small mobile phones and data transmission, typically used for radio and TV transmission

LEO systems

Orbit ca. 500 - 1500 km above earth surface

- visibility of a satellite ca. 10 - 40 minutes

- global radio coverage possible

- latency comparable with terrestrial long distance 
connections, ca. 5 - 10 ms

- smaller footprints, better frequency reuse

- Shorter life time , routing required

- but now handover necessary from one satellite to another

- many satellites necessary for global coverage

- more complex systems due to moving satellites

Examples:

Iridium (start 1998, 66 satellites)

  - Bankruptcy in 2000, deal with US DoD (free use, 
saving from “deorbiting”)

Globalstar (start 1999, 48 satellites)

  - Not many customers (2001: 44000), low stand-by times for mobiles

MEO systems:

Orbit ca. 5000 - 12000 km above earth surface

comparison with LEO systems:

- slower moving satellites

- less satellites needed

- simpler system design

- for many connections no hand-over needed

- higher latency, ca. 70 - 80 ms

- higher sending power needed

- special antennas for small footprints needed

Example:

ICO (Intermediate Circular Orbit, Inmarsat) start ca. 2000

  - Bankruptcy, planned joint ventures with Teledesic, Ellipso – cancelled again, start planned for 2003

- With a neat diagram explain the architecture of IEE 802.11 and also discuss the services provided by it.

Answer:

Categorizing Services

- Station services implemented in every 802.11 station

- Including AP stations

- Distribution services provided between BSSs

- May be implemented in AP or special-purpose device

- Three services used to control access and confidentiality

- Six services used to support delivery of MAC service data units (MSDUs) between stations

- Block of data passed down from MAC user to MAC layer

- Typically LLC PDU

- If MSDU too large for MAC frame, fragment and transmit in series of frames (see later)

Distribution of Messages Within a DS

- Distribution is primary service used by stations to exchange MAC frames when frame must traverse DS

- From station in one BSS to station in another BSS

- Transport of message through DS is beyond scope of 802.11

- If stations within same BSS, distribution service logically goes through single AP of that BSS

- Integration service enables transfer of data between station on 802.11 LAN and one on an integrated 802.x LAN

- Integrated refers to wired LAN physically connected to DS

- Stations may be logically connected to 802.11 LAN via integration service

- Integration service takes care of address translation and media conversion

Association Related Services

- Purpose of MAC layer transfer MSDUs between MAC entities

- Fulfilled by distribution service (DS)

- DS requires information about stations within ESS

- Provided by association-related services

- Station must be associated before communicating

- Three transition types of based on mobility

- No transition: Stationary or moves within range of single BSS

- BSS transition: From one BSS to another within same ESS

- Requires addressing capability be able to recognize new location

- ESS transition: From BSS in one ESS to BSS in another ESS

- Only supported in sense that the station can move

- Maintenance of upper-layer connections not guaranteed

- Disruption of service likely

Station Location

- DS needs to know where destination station is

- Identity of AP to which message should be delivered

- Station must maintain association with AP within current BSS

- Three services relate to this requirement:

- Association: Establishes initial association between station and AP

- To make identity and address known

- Station must establish association with AP within particular BSS

- AP then communicates information to other APs within ESS

- Reassociation: Transfer established association to another AP

- Allows station to move from one BSS to another

- Disassociation: From either station or AP that association is terminated

- Given before station leaves ESS or shuts

- MAC management facility protects itself against stations that disappear without notification

Access and Privacy Services - Authentication

- On wireless LAN, any station within radio range other devices can transmit

- Any station within radio range can receive

- Authentication: Used to establish identity of stations to each other

- Wired LANs assume access to physical connection conveys authority to connect to LAN

- Not valid assumption for wireless LANs

- Connectivity achieved by having properly tuned antenna

- Authentication service used to establish station identity

- 802.11 supports several authentication schemes

- Allows expansion of these schemes

- Does not mandate any particular scheme

- Range from relatively insecure handshaking to public-key encryption schemes

- 802.11 requires mutually acceptable, successful authentication before association

Access and Privacy Services - Deauthentication and Privacy

- Deauthentication: Invoked whenever an existing authentication is to be terminated

- Privacy: Used to prevent messages being read by others

- 802.11 provides for optional use of encryption